Allied Systems Integrations

One Source  |  Unlimited Solutions

P R O P O S A L

Access Control System Assessment & Smart Migration

Diamond Heights Village HOA

Prepared for:

Diamond Heights Village Association Board of Directors

c/o PMP Management LLC

Attn: Melissa Tolerton

Prepared by:

Don Bucknor, CEO — ASI 360

March 18, 2026

Proposal #: DHV-PROP-2026-004

CONFIDENTIAL — Valid 30 Days

# Cover Letter

March 18, 2026

Melissa Tolerton, Property Manager

PMP Management, LLC

Diamond Heights Village Association

115 Red Rock Way, San Francisco, CA 94131

Dear Ms. Tolerton,

Thank you for the opportunity to assess the access control system at Diamond Heights Village. Following our comprehensive remote assessment in March 2026 — including a detailed photo walkthrough of all controllers, readers, and infrastructure — we have developed a tailored solution designed specifically for DHV’s needs and budget.

Your existing system is built on proven Keri Systems hardware that is still functional. All 10 doors unlock without delay, all readers respond to credentials, and all four PXL-500 controller boards show active status LEDs. The core infrastructure works — what’s missing is the management workstation needed to add, remove, and modify resident credentials.

Rather than replacing equipment that functions properly, our Smart Migration approach restores management capability immediately, adds modern keypad and mobile credential access at two key entry points, and provides a frictionless transition plan for all 396 units — without disrupting a single resident’s access on day one.

ASI 360 is the right partner for this project for the following reasons:

- We are a Keri Systems authorized integrator with direct access to dealer pricing, technical support, and firmware resources — no middleman markup.

- Our hybrid NXT + Borealis architecture provides failover redundancy that an all-cloud solution cannot — if the internet goes down, 8 doors continue operating locally.

- We include a USB enrollment reader and fob recycling program, saving thousands in unnecessary credential purchases by reusing existing fobs.

- Our migration hotline and business card PIN distribution eliminates the “day-one chaos” of transitioning 700+ residents simultaneously.

We look forward to presenting this proposal at the Board Meeting on March 25th.

Respectfully,

Don Bucknor

CEO, ASI 360 — Allied Systems Integrations

don@asi360.co | (510) 495-0905

2342 Shattuck Ave. #866, Berkeley, CA 94704

CA Licensed · Bonded · Insured

# Table of Contents

Exhibit A: Problem Statement & Current Conditions4

Exhibit B: Equipment Inventory & Serial Registry6

Exhibit C: Reader Survey & Condition Assessment8

Exhibit D: Competitor Analysis9

Exhibit E: Proposed Solution — Hybrid Architecture10

Exhibit F: Fob Recycling & Credential Workflow12

Exhibit G: Investment Summary13

Exhibit H: Migration Timeline15

Exhibit I: Terms & Conditions16

Exhibit J: Acceptance17

# Exhibit A: Problem Statement & Current Conditions

## Client Statement (Verbatim)

“The computer we previously used to program new fobs has been misplaced. We are looking to upgrade to Keri Cloud Borealis and program 1,000 new fobs. Please see the attached redacted proposal under ‘Work Station’ and provide a similar proposal if possible. We would also appreciate any additional proposals with alternative options if you are able to prepare them; however, we would like to receive the Keri Cloud Borealis proposal as soon as possible in order to review it at the Board Meeting next Wednesday.” — Melissa Tolerton, Property Manager, PMP Management LLC

## Assessment Summary

Following our remote site assessment, ASI 360 identified the following conditions:

Current System Status:

- Keri Systems PXL-500 access control platform — 5 controller boards (4 standard + 1 Tiger master)

- All controllers manufactured 2018-2019 — within expected service life

- All 10 proximity readers powered and responding (LED indicators active)

- All 10 controlled doors unlock without delay when valid credentials are presented

- KPS-11 power supply with dual battery backup — functional

- Network connectivity via Ethernet to Tiger controller — active

The Root Cause:

- The management workstation has been misplaced — this is the ONLY component that is non-functional

- Without the workstation, PMP cannot: issue new fobs, deactivate lost/stolen fobs, modify access schedules, run audit reports, or manage any credential lifecycle operations

- The system hardware itself is fully operational

Key Finding: This is a management problem, not a hardware problem. Replacing functional readers and controllers is unnecessary and wasteful.

# Exhibit B: Equipment Inventory & Serial Registry

## Controller Registry

| # | Model | Serial Number | Manufactured | Condition | Action |
| --- | --- | --- | --- | --- | --- |
| 1 | PXL-500 | PXL500P190125385 | 2019 | Good — Green LEDs active | Retain |
| 2 | PXL-500 | PXL500P182524043 | 2018 | Good — Green LEDs active | Retain |
| 3 | PXL-500 | PXL500P190125385 | 2019 | Good — Green LEDs active | Retain (verify ≠ #1) |
| 4 | PXL-500 | PXL500P190125374 | 2019 | Good — Cables need tidying | Retain |
| 5 | PXL-500 (Tiger) | PXL500P190125373 | 2019 | Good — SB-593 satellite board | Retain as master |

## Power Infrastructure

| Component | Model | P/N | Condition | Action |
| --- | --- | --- | --- | --- |
| Power Supply | KPS-11 | 05279-001 | Good | Retain |
| Transformer | ESD X-1656 | — | Good — 16VAC 3.5A | Retain |
| Battery 1 | Interstate FAS1075 | — | Test on-site | Replace if >3 years |
| Battery 2 | Interstate FAS1075 | — | Test on-site | Replace if >3 years |

Note: Figure references correspond to photos submitted via the ASI 360 Field Intelligence Brief on March 2026.

# Exhibit C: Reader Survey & Condition Assessment

| Reader | Model | Mount Location | Condition | LED | Action |
| --- | --- | --- | --- | --- | --- |
| 1 | MS-5000 Mullion | Wood/stucco exterior | Good | Green | Retain |
| 2 | NXT-2R/3R | Wood doorframe | Poor — UV yellowed | Amber | Replace with CSR-6.2L |
| 3 | NXT-2R/3R | Stucco exterior | Poor — severe UV, dirty | Amber | Replace with CSR-6.2L |
| 4 | MS-5000 Mullion | Stucco near gate | Fair — scratched | Off/dim | Retain |
| 5 | MS-5000 Mullion | Stucco near gate | Fair | Off/dim | Retain |
| 6 | MS-5000 Mullion | Stucco near railing | Good | Amber | Retain |
| 7 | MS-5000 Mullion | Metal gate frame | Fair — weathered | Amber | Retain |
| 8 | MS-5000 Mullion | Metal gate frame | Fair — weathered | Amber | Retain |
| 9 | MS-5000 Mullion | Metal gate frame | Fair — edge wear | Amber | Retain |
| 10 | MS-5000 Mullion | Metal gate frame | Fair — debris | Amber | Retain |

Summary: 8 of 10 readers are functional and should be retained. Only Readers 2 and 3 (NXT-2R/3R type) show significant UV degradation requiring replacement. These two locations are recommended for upgrade to CSR-6.2L Connekt Smart Reader/Keypad units, adding keypad PIN and mobile credential capability at the main clubhouse entry and pool access point.

# Exhibit D: Competitor Analysis

| Feature | Competitor A | ASI 360 |
| --- | --- | --- |
| Approach | Replace all — cloud only | Smart Migration — hybrid |
| Readers replaced | All 10 (8 functional) | 2 only (UV damaged) |
| Entry methods | 1 (fob) | 3 (fob + mobile + keypad) |
| Mobile credentials | Not offered | 200 Connekt included |
| Keypad PIN codes | Not offered | At main entry + pool |
| Local management | None — cloud only | Workstation (Doors.NET) |
| Fob strategy | Buy 1,000 new | Recycle existing via USB reader |
| Migration plan | Day-one cutover | 6-week gradual transition |
| Failover if internet down | No local backup | 8 doors run locally |
| Monthly recurring | $150/mo (10 doors) | $39.90/mo (2 doors) |
| Year 1 total | ~$15,000 | $11,453 |
| 5-year total | ~$22,200 | $14,649 |
| 10-year total | ~$33,000 | $18,243 |

# Exhibit E: Proposed Solution — Hybrid Architecture

## Architecture Overview

The ASI 360 Smart Migration uses a hybrid architecture where:

- 8 existing doors continue operating on the proven PXL-500 platform, managed locally via Doors.NET

- 2 key entry points (Main Clubhouse Entry + Pool Gate) upgrade to Keri Borealis cloud with Farpointe CSR-6.2L Connekt Smart Reader/Keypad units

- A single ASI 360-managed workstation provides unified management of both systems

- Neither system is a single point of failure — they back each other up

## Phase 1 — Restore Management (Immediate)

- Deploy ASI 360 managed workstation with Doors.NET software

- Connect to existing PXL-500 controllers via LAN-520AESP Ethernet module

- Extract all existing credentials from controller memory

- Build resident database from PMP records

- Install USB enrollment reader for fob recycling

- System becomes immediately manageable again

## Phase 2 — Add Modern Access at Key Doors (Weeks 3-4)

- Replace UV-damaged Readers 2 and 3 with CSR-6.2L Connekt Smart Reader/Keypad

- Install Borealis Hub for cloud management of 2 doors

- Enable three entry methods at main entry + pool: fob tap, mobile phone, keypad PIN

- Begin mobile credential enrollment via email

- Distribute business cards with temporary PIN codes to all 396 units

## Future Expansion (When Budget Allows)

- Add Borealis doors incrementally: $555/reader + $19.95/door/month

- Today: 8 NXT + 2 Borealis = $479/yr

- Future: 6 NXT + 4 Borealis = $958/yr

- Full cloud: 0 NXT + 10 Borealis = $2,394/yr

- Board controls the pace — no pressure to go all-cloud

# Exhibit F: Fob Recycling & Credential Management Workflow

## The Transition Challenge

396 units × ~1.8 residents per unit = approximately 700 credential holders. Transitioning all 700 simultaneously is disruptive and expensive. Our approach:

## Fob Recycling Process

- Resident visits PMP office or “Fob Swap” event

- Staff plugs USB enrollment reader into workstation

- Scan existing fob → Doors.NET reads credential ID

- If resident is moving out: deactivate credential, fob enters recycle pool

- If new resident: assign recycled fob, program in Doors.NET, test at reader

- Cost per recycled fob: $0 (vs. $10.40 for new fob from stock)

## Business Card PIN Distribution

- Generate unique 6-digit PIN per unit from Doors.NET

- Print on professional business cards (1,000 cards = ~$150)

- Deliver to all units via door-drop or PMP office pickup

- PIN works immediately at CSR-6.2L readers (Main Entry + Pool)

- Valid for 30 days — renewable or converted to permanent

- Residents who prefer mobile: email enrollment link → phone becomes credential

## Credential Options per Resident

| Method | How They Get It | Works At | Cost | Notes |
| --- | --- | --- | --- | --- |
| Existing fob | Already have it | All 10 doors | $0 | No action needed |
| Recycled fob | Swap at office | All 10 doors | $0 | From recycling pool |
| New fob | From stock | All 10 doors | $10.40 | If recycled unavailable |
| Keypad PIN | Business card | 2 key doors | $0 | Immediate access |
| Mobile phone | Email enrollment | 2 key doors | $10/yr | Connekt app |

# Exhibit G: Investment Summary

## Hardware & Equipment

| # | SKU | Description | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- | --- |
| 1 | DELL-OPT7050 | ASI 360 Keri Hub — Managed Workstation (Dell i7/32GB/1TB) | 1 | $625.00 | $625.00 |
| 2 | KERI-DNET-BASIC | Doors.NET Basic Edition Software License | 1 | $495.00 | $495.00 |
| 3 | SW-PGWM | Keri Access Control Gateway License (ASI 360 remote support) | 1 | $750.00 | $750.00 |
| 4 | LAN-520AESP | Ethernet Communication Module | 1 | $385.00 | $385.00 |
| 5 | USB-ENROLL | USB Desktop Enrollment Reader (fob recycling) | 1 | $95.00 | $95.00 |
| 6 | CSR-6.2L | Farpointe Connekt Smart Reader/Keypad — Mullion (BLE+Prox+PIN) | 2 | $555.00 | $1,110.00 |
| 7 | KB-HUB | Keri Borealis Cloud Hub Controller | 1 | $574.00 | $574.00 |
| 8 | CMC-2 | Connekt Mobile Phone Credential (annual license) | 200 | $10.00 | $2,000.00 |
| 9 | 12V-7AH-SLA | Replacement Backup Battery (12V 7Ah SLA) | 2 | $36.50 | $73.00 |
|  |  | Hardware Subtotal |  |  | $6,107.00 |
|  |  | CA Sales Tax (8.625%) |  |  | $526.73 |
|  |  | Hardware Total |  |  | $6,633.73 |

## Services & Installation

| # | Item | Description | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- | --- |
| 1 | Data Extraction | Pull all existing credentials from PXL-500 controllers | 1 | $450.00 | $450.00 |
| 2 | Workstation Setup | Doors.NET install, network config, LAN module, USB reader | 1 | $950.00 | $950.00 |
| 3 | Reader Install | Remove old NXT-2R/3R, mount CSR-6.2L, terminate wiring | 2 | $185.00 | $370.00 |
| 4 | Borealis Config | Hub install, Connekt mobile setup, enrollment templates | 1 | $750.00 | $750.00 |
| 5 | Database Build | Import resident CSV, clean data, assign facility codes, PINs | 1 | $500.00 | $500.00 |
| 6 | Migration Support | Business card design, printing (1,000), distribution coordination | 1 | $300.00 | $300.00 |
| 7 | Training | PMP staff training — 2 sessions (Doors.NET + Borealis portal) | 1 | $500.00 | $500.00 |
| 8 | PM + Documentation | Project management, as-builts, migration report | 1 | $400.00 | $400.00 |
|  |  | Services Total |  |  | $4,220.00 |

## Annual Recurring

| Item | Description | Annual Cost | Monthly |
| --- | --- | --- | --- |
| BOREALIS-1D-MO | Borealis Cloud Subscription (2 doors × $19.95/mo) | $479.04 | $39.92 |
| TWILIO-LINE | Migration Hotline (optional, Year 1 only) | $120.00 | $10.00 |
|  | Total Annual Recurring | $599.04 | $49.92 |

COMBINED YEAR 1 TOTAL:  $11,452.77

(Includes hardware with tax, all services, and first-year subscriptions. Monthly recurring of $49.92 begins upon commissioning.)

# Exhibit H: Migration Timeline

| Week | Activities | Milestone |
| --- | --- | --- |
| 1 | Board approval → workstation procurement → credential extraction | System manageable |
| 2 | Resident CSV import → PIN generation → business card design & print | Database ready |
| 3 | CSR-6.2L reader install → Borealis Hub commissioning | Modern access live |
| 4 | Business card distribution → mobile enrollment email blast → Fob Swap event | Residents transitioning |
| 5 | Follow-up calls → hotline active → chase stragglers | 80%+ migrated |
| 6 | Final cleanup → deactivate unclaimed → as-built docs → project closeout | 100% complete |

# Exhibit I: Terms & Conditions

## 1. Payment Terms

- 40% upon signed acceptance ($4,581.11)

- 30% upon system commissioning ($3,435.83)

- 30% upon project completion ($3,435.83)

## 2. Warranty

- 1-year labor warranty on all installation work

- Manufacturer warranty on all equipment (Keri: 2 years, Farpointe: lifetime, Dell: 1 year)

## 3. Validity

This proposal is valid for 30 days from date of issue (expires April 17, 2026).

## 4. Scope Exclusions

- Structural modifications, fire alarm integration, elevator controls

- Door hardware replacement (hinges, closers, strikes)

- Exterior painting or surface restoration

## 5. Timeline

6 weeks from signed acceptance and deposit to project completion, subject to equipment availability.

## 6. Recurring Costs

Borealis subscription begins upon hub commissioning. Connekt mobile credentials are annual licenses, renewable.

## 7. Change Orders

Any work outside the scope described herein will be quoted separately and requires written approval.

## 8. Insurance

ASI 360 maintains general liability, workers’ compensation, and professional liability insurance. Certificates available upon request.

# Exhibit J: Acceptance

By signing below, the authorized representative of Diamond Heights Village Association accepts this proposal and authorizes ASI 360 to proceed with the described scope of work.

Total Investment: $11,452.77 (Year 1, inclusive of hardware, services, and first-year subscriptions)

Deposit Due Upon Signing: $4,581.11 (40%)

ACCEPTED AND AGREED:

Diamond Heights Village Association

Authorized Signature: _________________________________________     Date: ________________

Printed Name: ________________________________________________

Title: ________________________________________________________

ASI 360 — Allied Systems Integrations

Authorized Signature: _________________________________________     Date: ________________

Don Bucknor, CEO

don@asi360.co | (510) 495-0905